Smart Automatic Toll Gate System Using Arduino & RFID

Create an automated toll gate with Arduino that detects vehicles, verifies RFID payment cards, deducts tolls, and controls a servo-driven gate seamlessly.

# Components and supplies

- 1 Ring with 12 RGB WS2812 LEDs and integrated driver

- 1 RS422 / RS485 Shield for Arduino UNO

- 1 Servo Motor SG90 180 degree

- 1 RFID Reader Module

- 1 ir sensor

# Project Overview

This is a Smart $ Automatic Toll Gate System Project Using Arduino  $ , RFID cards, and basic sensor modules to automate toll collection. Designed for learning and real-world relevance, this system detects vehicles, authenticates payments via RFID, opens a servo-controlled toll barrier, and resets for the next vehicle — all without manual intervention.

This toll gate automation demonstrates how embedded systems and low-cost components can transform traditional toll booths into contactless, efficient, and scalable solutions — ideal for student projects, IoT demos, and smart city prototypes.

How It Works

The automated toll system operates through a clear, event-driven workflow:

- Vehicle Detection

An IR sensor at the entrance detects an approaching vehicle and triggers the system to start listening for an RFID card.

- RFID Authentication

Drivers tap their RFID card near the RC522 reader. The Arduino reads its unique ID and checks if it’s stored in the onboard database with sufficient balance.

- Validation Logic

Valid card + sufficient balance: Toll is deducted automatically.

Invalid card / no balance: Access is denied; red LED lights up.

Unregistered card: Rejected with error feedback.

- Gate Control

Upon successful payment:

Green LED turns on.

The servo motor rotates to open the gate (~90°).

On exit detection by the second IR sensor, the gate closes automatically.

The flow repeats for subsequent vehicles without resetting the Arduino manually.

# Wiring & Connections

Below is a concise reference for wiring major components:

RFID RC522 Reader

| RFID Pin | Arduino |
| --- | --- |
| SDA | D10 |
| SCK | D13 |
| MOSI | D11 |
| MISO | D12 |
| RST | D9 |
| 3.3V | 3.3V |
| GND | GND |

IR Sensors

| IR Sensor | Arduino |
| --- | --- |
| Entry Sensor | OUT D2 |
| Exit Sensor | OUT D3 |
| VCC | 5V |
| GND | GND |
| LED | Indicators |
| LED | Arduino |
| Red | D6 |
| Green | D7 |
| GND | GND |

Servo Motor

| Servo Pin | Arduino |
| --- | --- |
| Signal | D5 |
| VCC | 5V |
| GND | GND |

All sensors share a common ground. The RFID uses 3.3V logic, so confirm proper voltage to prevent damage.

# Arduino Code Highlights

Your Arduino sketch includes:

- Libraries: SPI (communication), MFRC522 (RFID), Servo (gate control)

- Pin Definitions & Setup: Map hardware pins to intuitive constants

- Main Loop: Listen for vehicle presence (IR) and RFID card

- Authenticator: Matches UID against allowed cards and checks balance

- Servo Control: Opens/closes the gate and updates LEDs accordingly onde ele está. Basta usar a opção Personalizar Sumário para fazer esse tipo de alteração e o Word fará a alteração.

#include <SPI.h>

#include <MFRC522.h>

#include <Servo.h>

#define SS_PIN 10

#define RST_PIN 9

#define IR_ENTRY 2

#define IR_EXIT 3

#define LED_RED 6

#define LED_GREEN 7

#define SERVO_PIN 5

Servo gateServo;

MFRC522 mfrc522(SS_PIN, RST_PIN);

void setup() {

Serial.begin(9600);

SPI.begin();

mfrc522.PCD_Init();

gateServo.attach(SERVO_PIN);

pinMode(IR_ENTRY, INPUT);

pinMode(IR_EXIT, INPUT);

pinMode(LED_RED, OUTPUT);

pinMode(LED_GREEN, OUTPUT);

gateServo.write(0); // Start with gate closed

}

More logic handles UID comparison, balance update, and event sequencing.

Real-World Applications

Beyond a classroom project, this architecture has practical relevance:

- Highway Toll Plazas: Faster and contactless tolling.

- Parking Management: Automated entry and exit tracking.

- Gated Communities: Efficient resident vehicle access.

- Industrial Facilities: Track truck movement and permit access.

Future Enhancements

Once the base system is working, you can expand it with:

- LCD Display: Show balances, messages, and system status.

- Database Logging: Store transaction history on SD or cloud (ESP32/Firebase).

- Mobile App Link: View and manage balances remotely.

- OCR License Plate Recognition: Link vehicles to RFID identity.

- Solar Power Integration: Make toll booths energy independent.

# Downloadable files

https://github.com/VedhathiriK/Built-Your-Own-GPS-Tracker-with-Geofence-Using-Xiao-ESP32-S3-/archive/refs/heads/main.zip

# References

https://projecthub.arduino.cc/rinme/smart-automatic-toll-gate-system-using-arduino-rfid-105b72