RFID-RC522 Sensor

It senses your card/keychain and then turns on either a green or red led.

Este documento foi desenvolvido por Andrei Inoue Hirata está protegido por direitos autorais, sendo vedada sua reprodução sem autorização prévia do autor.

# Components and supplies

- 1 Arduino Uno Rev3

- 1 5 mm LED: Green

- 1 5 mm LED: Red

- 1 RFID Module (Generic)

- 11 Jumper wires (generic)

- 1 RFID Module (Generic)

- 1 Breadboard (generic)

# Project description

I am trying to make a lock for my door like the ones at hotels, and also the leds the green one goes to pin 7 and red goes to pin 6 on the big power side (longer side) and the other side (smaller side) goes to GND

Code

#include <SPI.h>

#include <MFRC522.h>

#define SS_PIN 10

#define  RST_PIN 9

#define RED 2

#define Green 3

MFRC522 mfrc522(SS_PIN, RST_PIN);   // Create MFRC522 instance.

void setup()

{

Serial.begin(9600);   // Initiate a serial communication

SPI.begin();      // Initiate  SPI bus

mfrc522.PCD_Init();   // Initiate MFRC522

Serial.println("Approximate your  card to the reader...");

Serial.println();

pinMode(LED_BUILTIN, OUTPUT);

digitalWrite(LED_BUILTIN, LOW);    // turn the LED off by making the voltage  LOW

digitalWrite(Green, LOW);

digitalWrite(RED, LOW);

}

void  loop()

{

// Look for new cards

if ( ! mfrc522.PICC_IsNewCardPresent())

{

return;

}

// Select one of the cards

if ( ! mfrc522.PICC_ReadCardSerial())

{

return;

}

//Show UID on serial monitor

Serial.print("UID  tag :");

String content= "";

byte letter;

for (byte i = 0; i <  mfrc522.uid.size; i++)

{

Serial.print(mfrc522.uid.uidByte[i] < 0x10  ? " 0" : " ");

Serial.print(mfrc522.uid.uidByte[i], HEX);

content.concat(String(mfrc522.uid.uidByte[i]  < 0x10 ? " 0" : " "));

content.concat(String(mfrc522.uid.uidByte[i],  HEX));

}

Serial.println();

Serial.print("Message : ");

content.toUpperCase();

if ((content.substring(1) == "1A 7C 78 30") || (content.substring(1) == "CC  CC A3 16") ) //change here the UID of the card/cards that you want to give access

{

Serial.println("Authorized access");

Serial.println();

digitalWrite(LED_BUILTIN,  HIGH);   // turn the LED on (HIGH is the voltage level)

digitalWrite(Green,  HIGH);

delay(10000);

digitalWrite(Green, LOW);

}

else   {

Serial.println(" Access denied");

digitalWrite(LED_BUILTIN,  LOW);    // turn the LED off by making the voltage LOW

digitalWrite(RED,  HIGH);

delay(10000);

digitalWrite(RED, LOW);

}

}

#include <SPI.h>

#include <MFRC522.h>

#define SS_PIN 10

#define

RST_PIN 9

#define RED 2

#define Green 3

MFRC522 mfrc522(SS_PIN, RST_PIN);

// Create MFRC522 instance.

void setup()

{

Serial.begin(9600);

// Initiate a serial communication

SPI.begin();      // Initiate  SPI bus

mfrc522.PCD_Init();   // Initiate MFRC522

Serial.println("Approximate your

card to the reader...");

Serial.println();

pinMode(LED_BUILTIN, OUTPUT);

digitalWrite(LED_BUILTIN, LOW);    // turn the LED off by making the voltage

LOW

digitalWrite(Green, LOW);

digitalWrite(RED, LOW);

}

void

loop()

{

// Look for new cards

if ( ! mfrc522.PICC_IsNewCardPresent())

{

return;

}

// Select one of the cards

if ( ! mfrc522.PICC_ReadCardSerial())

{

return;

}

//Show UID on serial monitor

Serial.print("UID

tag :");

String content= "";

byte letter;

for (byte i = 0; i <

mfrc522.uid.size; i++)

{

Serial.print(mfrc522.uid.uidByte[i] < 0x10

? " 0" : " ");

Serial.print(mfrc522.uid.uidByte[i], HEX);

content.concat(String(mfrc522.uid.uidByte[i]

< 0x10 ? " 0" : " "));

content.concat(String(mfrc522.uid.uidByte[i],

HEX));

}

Serial.println();

Serial.print("Message : ");

content.toUpperCase();

if ((content.substring(1) == "1A 7C 78 30") || (content.substring(1) == "CC

CC A3 16") ) //change here the UID of the card/cards that you want to give access

{

Serial.println("Authorized access");

Serial.println();

digitalWrite(LED_BUILTIN,

HIGH);   // turn the LED on (HIGH is the voltage level)

digitalWrite(Green,

HIGH);

delay(10000);

digitalWrite(Green, LOW);

}

else

{

Serial.println(" Access denied");

digitalWrite(LED_BUILTIN,

LOW);    // turn the LED off by making the voltage LOW

digitalWrite(RED,

HIGH);

delay(10000);

digitalWrite(RED, LOW);

}

}

# Schematics

Xxxxxxxx

# Referencias

https://projecthub.arduino.cc/person87/rfid-rc522-sensor-64000f